# SOP-SAFE-007: Hot Work Permit

SAMPLE DOCUMENT — fictional "Sample Refinery Ltd." — created for demonstrating the Sovereign Workbench prototype only. Values are illustrative.

Revision: 4 | Effective date: 10-02-2026 | Owner: Fire and Safety Department

# 1. Purpose

To control any work that produces a flame, spark or heat high enough to ignite flammable material, such as welding, gas cutting, grinding and drilling with non-spark-proof tools, inside the refinery battery limits.

# 2. Who Issues the Permit

The permit is issued by the Area Shift In-charge (issuer) to the Job Executing Engineer (receiver). A Fire and Safety officer must countersign every hot work permit inside hazardous areas.

# 3. Gas Test Before Work

A gas test must be done with a calibrated multi-gas detector immediately before work starts and repeated every 2 hours during the work. Work may start only if all of the following limits are met:

| Parameter | Acceptable limit |
| --- | --- |
| Flammable gas (LEL) | 0 % LEL |
| Oxygen | 19.5 % to 23.5 % |
| Hydrogen sulphide (H2S) | Below 1 ppm |
| Carbon monoxide (CO) | Below 25 ppm |

# 4. Precautions

All drains and sewers within 15 metres must be covered with wet gunny bags. A fire watcher with a fire extinguisher must be present throughout the work and for 30 minutes after it ends. Equipment must be isolated with blinds (not only valves) and depressurised.

# 5. Validity

A hot work permit is valid for one shift only (maximum 8 hours) and must be renewed for the next shift after a fresh gas test. The permit must be displayed at the work site.